sbRIO-9626 SoftMotion Servo Control Demo

HW/SW Requirements:

- sbRIO-9626

- NI 9694 RMC Breakout Board (or some other RMC breakout board)

- Servo Motor and Drive

- LabVIEW 2014, Real-Time, FPGA, SoftMotion Modules

This demo supports up to 5 axes, but sbRIO-9626 only has 4 AO channels.  Code can be easily compiled for other targets.

Signal Connections:

| sbRIO-9626 | Servo Motor |
| --- | --- |
| DIO 60 | ENC A+ |
| GND | ENC A- |
| DIO 42 | ENC B+ |
| GND | ENC B- |
| AO 0 | Vel. Voltage Input |
| AGND | Vel. Voltage GND |
| DIO 29 | Reverse Limit (optional) |
| DIO 93 | Forward Limit (optional) |
| DIO 11 | Home Limit (optional) |

*Servo ON is not used in this example.  You will need the set Servo ON from the drive manually.

Running the example:

- Open “5-Axis Servo Test Panel.vi” from “5-Axis 9626 Servo Control.lvproj”.  Set the PID Gains all to 0, then run this VI.

- Check the encoder feedback by moving the motor by hand.  You should see the number in “Position Feedback” change in the direction of the turn.

- Check the analog output by pressing the “AO Override” button.  You can adjust the output voltage manually in the “AO Manual” control.  The motor should turn with a speed proportionally to the voltage you set.  When you are done, press the “Reset Encoder 1” and turn off the “AO Override” button.

- Tune the PID of the motor by adjusting the PID gains.  Start adjusting kp with a low number (e.g. 0.1.)  Continue with ki and kd until the motor is stable.  If the motor is unstable and “pos error has exceeded” (Boolean indicator), reset the PID engine and encoder by repeating step 3.

- Now you can execute moves using the parameters from the front panel.  You can select the move type and press the “Execute Command” button.  “Stop all” will stop all motors.

John Wu

NI Taiwan System Engineering

wei-han.wu@ni.com

12/10/2014